TheAdminPage

"DSL Servers"

Setting Up A DSL Server

By: QAPete

What's DSL?

DSL (Digital Subscriber Line) comes in a couple different flavors. Most end users have what is called ADSL

(Asymmetric Digital Subscriber Line), which can transmit up to 6 Mb/sec to your computer, and up to about

1.5 Mb/sec from your computer. SDSL (Symmetric Digital Subscriber Line) can supply about a 1.5 Mb/sec

pipe to and from your computer. Both services are limited in range from your local phone company switching

office, with a max of about 12,000 feet. Your service will deteriorate near the end of that limitation, and not

all areas can get DSL service.

DSL is basically an invisible network connection to your provider. When you power on your computer, you're

online. Period. No dialing up, no busy signals, incredible speed. All the bandwidth you purchase is yours,

unlike cable, where you end up sharing with your neighbors on a token ring network. This has major

advantages when running a Unreal Tournament server. Prices for DSL vary and are changing all the time, but

generally you're going to pay about $50-$80 a month, plus any equipment and setup fees your provider

requires. The speed difference you'll see, though, is staggering.

Setting Up

Setting up a DSL server is quite similar to a cable server, although you're going to have all that nice

bandwidth dedicated for your users, and you are likely going to be able to support them better via DSL.

Rather than repeat what is already available on the Cable Modem Servers page, I'll concentrate on the

DSL-specific things.

How Many Users?

A common question, and I'll give you my answer. Let's say, for this example, you have a 768/384 KB/sec

DSL line. What I suggest you do is divide your outgoing bandwidth by 28.8 KB/sec, then multiply that result

by .9. Based on your bandwidth alone, you should be able to comfortably run a dedicated (not listen) server

for 12 people. Going with a higher max players is only going to slow down performance for all players.

There's another important issue here - CPU speed. From the old scientific method of trial and error, plus my

experiences with my own and other people's servers, you should be able to stick with the max users using

the formula above if you are running a Celeron (or P2) 300 mhz or better for most maps. Anything slower,

and you run the risk of slowdowns based on your processor. I'd also set the Game Engine cache to 40 mb

or 1/2 of your total RAM, whichever is higher. This will reduce swapping to the drive, which will completely

destroy your server performance.

Pings Good, Performance Sucks

Typical complaints, sometimes your issue, sometimes not. First, your provider. You're going to have to see,

based on your own experiences online, if your provider is responsible for packet loss, multiple retries for

packets, etc. There's just no subtitute for a good provider. In a DOS window, do a tracert xxx.xxx.xxx.xxx

(use known IP or internet addresses) to see the routes your provider has to different geographical areas of

the country - see how many timeouts you get (some of these will be normal, as many routers do not

respond to this command, unfortunately). Be aware of actual computers on the internet in your tracert and

any problems with times to respond.

Another nasty thing many DSL owners running Win95/98 come across is something called RWIN. This is a

registry setting that you simply MUST set correctly for your DSL to operate properly (affects the download

side, not the upload side). To set up your RWIN value to the correct one, follow these instructions

exactly (use at your own risk - editing the registry is tricky business if you're not careful:

Run REGEDIT from the Start menu (Start Menu | Run | regedit.exe)

Find the section HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\VxD\MSTCP

After left clicking ONCE on this section, look at the right pane. You should see several keys there. If

there is a key there called DefaultRcvWindow, update it to 65535 by double-clicking the value at the

right-hand side. If there is no key, create one as a 'String Value', and put 65535 in it.

Close REGEDIT, it will automatically save your changes.

You will have to restart your computer for the changes to take effect.

Remember - the RWIN value is the ONLY one you will need to set. Win9x generally defaults this value to

8k, which is unsatisfactory for DSL (and UT server) performance. Don't be messing with other keys, or you're

going to mess up your computer but good (and I'll have to biotch slap you).

Windows NT users need not worry about this value, as NT is generally pretty good about dynamically tuning

TCP/IP performance, along the lines of Linux and other non-Win9x operating systems. For more information

about that, check out this page.

Geography counts. If someone from Mongolia is trying to access your St. Louis-based DSL server,

they're going to have a nasty time of it. Period. Pings and packet loss will likely make it impossible for them

to play well on your server, and there's nothing you can do to improve that. Suggest to the user that they

find a server located closer to them geographically.

I Want More DSL Information

While there are a lot of good DSL info pages on the web, I highly recommend going to DSL Reports, the

most comprehensive of the bunch. If it's not there, it probably doesn't exist.

UCC or UT?

While this is not strictly DSL-oriented, it's an option you have that people don't really talk about, including

the official Unreal Techology page. My man Mutant Killer likes using UCC, for several reasons. I opt for

using unrealtournament.exe, and I have my reasons. First, you get the taskbar icon, which allows you to

pull up the console, advanced options and quickly quit the server. Second, it's just as easy to start up the

server using a desktop shortcut. Here one you can use for a team DM game:

c:/unrealtournament/system/unrealtournament.exe

DM-Codex.unr?Game=BotPack.TeamGamePlus?Mutator=?Listen -server log=server.log

If you have multiple ports you wish to use, just make copies of your unrealtournament.ini file, edit them by

changing the port=7777 to whatever you want, and save them with new names. Now you can add the

ini=otherinifile.ini option just before the log=server.log portion of the above command line. Don't use the

port=xxxx command right in the command line, as I've seen the server get cornfused, as it reads the .ini file

again, overwriting your choice of port when it changes levels. Ack.

If you have any questions, or wish for me to add to/change/update this page with other information, just send

me an email at qapete@planetunreal.com and I'll be glad to help.

TheAdminPage

12/12/99